Ronobir Mookherjee

Design Assignment 5 Part 2

Description:

Similar to Part 1, but here the data from the temperature sensor is transferred to a 4 digit 7 Segment LED display driven from an LED driver chip, here the max 7221.

Flowchart:

Code:

/*

* CpE301DesignAssignment5Part2.c

*

* Submission for part 1

Part 1 and 2 were combined into one project, and are being separated and submitted individually.

The purpose of part 2 is to get the value of the Temperature using ADC once per second and outputting to a 7 segment display

The four primary components used include the Temperature sensor LM34 from TI,  the Atmega328p, the Maxim 7221cng, and a 4 Digit 7 Segment display

The transmission protocol used here is SPI

The MAX 7221 chip was employed because of its specially properties as listed: BCD decoder, Multilpex scan, SRAM array, and you only need 1 external resistor to provide the necessary power to the 4 digit 7 Segment display. The Maxim 7221 is known as an 8 digit LED display Driver.

*  Author: Ronobir Mookherjee

*/

/*  Below is the standard setup for an AVR project, with preprocessor directives*/

#include <stdlib.h>

#include <alloca.h>

#include <assert.h>

#include <ctype.h>

#include <errno.h>

#include <inttypes.h>

#include <math.h>

#include <setjmp.h>

#include <stdint.h>

#include <stdio.h>

#include <string.h>

#include <avr/boot.h>

#include <avr/io.h>

#include <avr/cpufunc.h>

#include <avr/eeprom.h>

#include <avr/fuse.h>

#include <avr/interrupt.h>

#include <avr/lock.h>

#include <avr/power.h>

#include <avr/pgmspace.h>

#include <avr/sfr_defs.h>

#include <avr/sleep.h>

#include <avr/version.h>

#include <util/crc16.h>

#include <util/parity.h>

volatile int TempRead;

volatile char strBuffer[4];

volatile char binZero = 0b00111111;

volatile char binOne = 0b00000110;

volatile char binTwo = 0b01011011;

volatile char binThree = 0b01001111;

volatile char binFour = 0b01100110;

volatile char binFive = 0b01101101;

volatile char binSix = 0b01111101;

volatile char binSeven = 0b00000111;

volatile char binEight = 0b01111111;

volatile char binNine = 0b01101111;

//Above is for debugging purposes, to send data each segment individually, each represents a number on the 7 segment display

void InitialzeIO(void){

// Debug ports to check if all functions are working

DDRC = (0 << DDC0); //Input Port for ADC Temp Sensor

/*PORTC |= (1<<PORTC0);*/

DDRD |= (1<<DDD2)|(1<<DDD6); // Debug Ports

}

void InitTimer(void) //Timer used for ADC conversion, set to 1s

{

OCR1A = 8000; //1s value at 8mhz clock rate

TIMSK1 |= (1 << OCIE1A); //enables CTC mode interrupts

TCCR1B |= (1 << WGM12); //sets to Mode 4 or CTC mode

}

void startTimer(void) //function call that starts timer

{

TCCR1B |= (1 << CS12) | (1 << CS10); //clock prescaler that divides clock 1024 for the system clock

}

void stopTimer(void) //stops timer

{

TCCR1B &= ~((1 << CS12) | (1 << CS10));

TIMSK1 &=~(1<<OCIE0A);

}

void InitADC(){ //initializes ADC

//select Vref = AVCC and left adjust result setting to 8 bits

ADMUX = (0<<REFS1)|(0<<REFS0)|(1<<ADLAR);

//set prescaler, enable autoriggering, enable adc interrupt, and enable adc

ADCSRA |= (1<<ADPS1)|(1<<ADPS2)|(1<<ADPS0)|(1<<ADATE)|(1<<ADIE)|(1<<ADEN);

ADCSRB |= (0<<ADTS1)|(1<<ADTS0)|(1<<ADTS2); //Set clock division value to 256

}

void SetADCChannel(int ADCchannel){ //function to select which ADC channel to pickup from for output going to the ADCMUX

if (ADCchannel == 5)

{

ADMUX = (0<<MUX1)|(0<<MUX3)|(1<<MUX2)|(1<<MUX0);

}

else if (ADCchannel == 0)

{

ADMUX = (0<<MUX1)|(0<<MUX3)|(0<<MUX2)|(0<<MUX0);

}

else

{

ADMUX = (0<<MUX1)|(1<<MUX3)|(0<<MUX2)|(0<<MUX0);

}

}

void startADC(void) //Starts ADC transmission

{

ADCSRA |= (1<<ADSC);

}

void StopADC(void)

{

ADCSRA &= ~((1<<ADEN)|(1<<ADIE));

}

void delayforLED(void)  // use Timer0 to generate 2.5 ms delay for 4 Digit 7 Segment Display

{

// Set the Timer Mode to CTC

TCCR0A |= (1 << WGM01);

// Set the value that you want to count to

OCR0A = 125;

// start the timer

TCCR0B |= (0 << CS01) | (0 << CS00)| (1 << CS02);

// set prescaler to 256 and start the timer

while ( (TIFR0 & (1 << TOV0) ) > 0)        // wait for the overflow event

{

}

TIFR0 &= ~(1 << TOV0);

// reset the overflow flag

}

void initializeSPI(void) //SPI communication used to communicate with the LED Driver Chip, Max 7221

{

DDRB |= (1<<2)|(1<<3)|(1<<5);    // SCK, MOSI and SS as outputs

DDRB &= ~(1<<4);                 // MISO as input

SPCR |= (1<<MSTR);               // Set as Master

SPCR |= (1<<SPR0)|(1<<SPR1);     // divided clock by 128

}

void startSPI(void) //Enables the start bit on the SPI register

{

SPCR |= (1<<SPE);                // Enable SPI

}

void sendDataSPI(unsigned char cmd, unsigned char data) //function to send data, first with the command op code, and then the data values to be displayed

{

PORTB &= ~(1<<2); // turn off slave select for transmission so there isn't any noise picked up

SPDR = cmd;                 // Command data, used by the Max 7221 for operation, such as selecting which digits, or how many digits

while(!(SPSR & (1<<SPIF)));  // wait until transmission is complete

SPDR = data;                 // send the data

while(!(SPSR & (1<<SPIF)));  // wait until transmission is complete

PORTB |= (1<<2); //enables slave select and finishes transmission

}

int binary_decimal(int n) /* Function to convert binary to decimal. used for debugging*/

{

int decimal=0, i=0, rem;

while (n!=0)

{

rem = n%10;

n/=10;

decimal += rem*pow(2,i);

++i;

}

return decimal;

}

void stopSPI(void)

{

SPCR |= ~(1<<SPE); //Disable SPI

DDRB &= ~((1<<2)|(1<<3)|(1<<5));   // SCK, MOSI and SS as inputs

DDRB |= (1<<4);                    // MISO as output

SPCR &= ~(1<<MSTR);                // Set as slave

}

void initialize7Seg(void) //initializes the 4 digit 7 segment display and the Maxim chip

{

sendDataSPI(0x09, 0b00001111); //Scans for 4 digits of the 7 segment display

sendDataSPI(0x0B, 0x04); // enables operation for 4 digits

sendDataSPI(0x0C, 0x01); // turns on the display

}

void updateDisplay(int Temperature)

{

//******* Remember that the Max 7221 chip has a built in BCD ********//

//for the first digit

sendDataSPI(0x01, Temperature/1000); //shifts first digit of data

sendDataSPI(0x02, ((Temperature % 1000)/ 100)); //shifts next digit, using mod of 10^3

sendDataSPI(0x03, ((Temperature%100)/10)); // shifts third digit

sendDataSPI(0x04, ((Temperature%100)%10)); //shifts final digit

// With the Max 7221 chip you don't need to refresh the display quickly to display all characters, the LED Driver takes care of this

}

int main(void) //setups up the chip, with the function calls to enable communication using SPI and ADC conversion

{

InitialzeIO();

InitADC();

InitTimer();

startTimer();

startADC();

initializeSPI();

initialize7Seg();

startSPI();

while(1)

{

//main loop

}

}

ISR(ADC_vect) // ADC to pull data from the temperature sensor every 1 second

{

while(ADIF == 0){

};//check to see if ADCH and ADCL have been populated before proceeding

TempRead = ADCH;// is the value of the temperature sensor captured and stored in the variable

TIFR1 = (0<<OCF1A)|(0<<OCF1B);//disables flag to reset interrupt and start again

StopADC();//stops ADC to restart back to main.

itoa(binary_decimal(TempRead), strBuffer, 10); //used for debugging, the itoa function takes the integer and stores in a string buffer (array) that can also be used to send data, digit by digit

updateDisplay(TempRead); //outputs to the 4 digit 7 segment display using SPI communication

stopSPI();

}

Schematics: